WRO Specification type

Document

< Specification and Certification TV Wall Outlet >

Document Housekeeping

Document Category and type

| CAT | TYPE | DOC ID | Comment |
| --- | --- | --- | --- |
| TV | SPEC | WRO_TA_A_S_PAAA | Specification type documents (-SPEC) are documents specifying logical / physical interfaces / protocols, etc.., to which AO equipment/systems need to comply |

Document Status

| EDITION | DATE | STATUS |
| --- | --- | --- |
| 1.0 | 14.07.2023 | Final |

Legal Disclaimer

“This document constitutes an integral part of the Wyre Reference Offer for Basic TV / IDTV / BB and should be fully complied with by the Beneficiary at all times. Non compliance, incomplete or deviating application of this document by the Beneficiary, or his authorized agent, results in the suspension and ultimately termination of the Contract between Wyre and the Beneficiary.

At any time this document is susceptible to change by Wyre, Regulator’s decision or by decision of a relevant judicial authority. Changes to this document will, depending on the circumstances for change, be appropriately notified to the Beneficiary and published on the Wyre website.

Table of Contents

1	Abstract	5

2	Wall Outlet Functional Description	6

3	Wall Outlet Functional Requirements	7

3.1	RF Requirements	7

3.1.1	Insertion Loss	7

3.1.2	Return Loss	7

3.1.3	TV to FM Signal Isolation	7

3.1.4	Group Delay	7

3.1.5	EMC Screening	7

3.2	AO Device Management by Wyre Requirements	8

3.2.1	Concept and purpose	8

3.2.2	Device management Functions	8

3.2.3	SNMP MIB specifications	8

3.2.4	Reset and Factory Reset specifications	8

4	Wall Outlet Non - Functional Requirements	9

4.1	Mechanical Requirements	9

4.1.1	Housing	9

4.1.2	Diagnostic Leds	9

4.1.3	Labels	9

4.1.4	TV and FM Radio connectors	9

4.2	Environmental Requirements	10

4.2.1	Packaging	10

4.2.2	RoHS and WEEE compliancy	10

4.2.3	EU CoC compliancy	10

4.3	Safety Requirements	10

4.3.1	Surge and Lightening protection	10

4.3.2	Temperature and Humidity	10

4.3.3	Fire resistance	10

4.4	EU Consumer Goods label Requirements	10

4.4.1	CE - mark	10

5	AO Equipment Wall Outlet Certification	11

5.1	Introduction	11

5.2	Test score card	11

Table of Figures

Figure 3-1: Insertion Loss	7

Figure 3-2: Return Loss	7

Figure 3-3: TV to FM Signal Isolation	7

Figure 3-4: Group Delay	7

Figure 3-5: EMC Screening	7

List of Appendixes

This document may refer to further detailed documents that are added in Appendixes to this document.

A reference to an appendix is in this document highlighted with grey background.

The list with appendixes of this document:

- None

List of References

This document may refer to external documents or information sources.

A reference to an external document or information source is in this document highlighted with grey background.

The list of referred external documents or information sources in this document:

Reference 1: WRO_TA_G_C_PAAA – General Certification Procedures

Restricted information

This document may contain sections that are not public information and that can be made available only to parties that have executed specific NDA`s.

Information that is subject to NDA is marked in this document as follows:

The information in this text box is available only under NDA

NDA

NDA

Before conversion to PDF format for publication of the document, the information will be made unreadable by converting the background of the text box to black.

# Abstract

This document describes functional specification, requirements and non-functional requirements for Wyre Wall Outlet.

Generic sections specifying certification procedures applicable to all AO CPE or network equipment that will be connected to the Wyre network are described in General Certification Procedures Document WRO_TA_G_C_PAAA – General Specification Procedures.

# Wall Outlet Functional Description

- The wall outlet (WO) is the termination point of the inbound coax cable at customer home which provides physical connectivity for CPEs. It contains connection points for analogue TV´s, digital STB´s and FM radio´s.

- In a customer home where no active NIU is present, the WO also serves as the signal termination and transfer point between the “outside” network and the “in-home” coax network. The outside network is brought into the home via a “drop” cable which originates from the “tap” point on the outside Wyre coax plant and terminates on a WO. This particular WO, which is the first in line WO, terminates the drop cable, is the transfer or hand-over point between the outside Wyre network and the customer in home network. It is possible that more WO´s are present in a customer´s home that are daisy chained downstream from the main (hand-over) WO. Those types of WO´s are not hand-over points and belong to the in-home network.

- In some cases also a hand-over connector or hand-over device may be present, in which case this device is the formal termination and hand-over point. Also in this case a WO will have to be present after this hand-over connector / device, so for the sake of keeping the technical architecture documents of manageable complexity, the first in line WO will be referred as performing the role of signal transfer point for TV and radio signals.

- In a customer home that is or will be enabled for interactive digital services requiring two way communications over the HFC network” a NIU is/will be present and as such the NIU is the termination and transfer point between the “outside” network and the “in-home” coax network. One or more WO´s will be connected to the downstream ports of the NIU. STB´s will be connected via a coax patch cord cable to such a WO as well as classical analogue TV sets or FM radio´s.

- All the different types of WO´s discussed above are technically the same and need to be compliant to the specification

# Wall Outlet Functional Requirements

## RF Requirements

### Insertion Loss

- Insertion Loss Requirement for Wall Outlet is shown in below figure.

Figure 3-1: Insertion Loss

### Return Loss

- Return Loss Requirement for Wall Outlet is shown in below figure.

Figure 3-2: Return Loss

### TV to FM Signal Isolation

- TV to FM Signal Isolation Requirement for Wall Outlet is shown in below figure.

Figure 3-3: TV to FM Signal Isolation

### Group Delay

- Group Delay Requirement for Wall Outlet is shown in below figure.

Figure 3-4: Group Delay

### EMC Screening

- EMC Screening Requirement for Wall Outlet is shown in below figure.

Figure 3-5: EMC Screening

## AO Device Management by Wyre Requirements

### Concept and purpose

- As this document describes the Wall Outlet Interface Specification, Concept and Purpose are not applicable.

### Device management Functions

- As this document describes the Wall Outlet Interface Specification, Device Management Functions are not applicable.

### SNMP MIB specifications

- As this document describes the Wall Outlet Interface Specification, SNMP MIB specifications are not applicable.

### Reset and Factory Reset specifications

- As this document describes the Wall Outlet Interface Specification, Reset and Factory Reset specifications are not applicable.

# Wall Outlet Non - Functional Requirements

## Mechanical Requirements

### Housing

- Full metal housing is required to be compliant with listed EMC screening requirements.

- The wall outlet´s should be suitable for build-in (must fit 60mm build-in wall box) or alternatively for build up (surface mounting). TV & FM ports shall be placed on front of the WO cover plate and shall be clearly labeled. The figures below show, as an example a typical design for a build-in WO.

TV

FM

### Diagnostic Leds

- The Wall Outlet does not include any diagnostic leds for troubleshooting purposes.

### Labels

- Following labels must be printed on the Wall Outlet after TELENET-INTERKABEL certification.

- TV

- FM

- TELENET INTERNETKABEL

### TV and FM Radio connectors

- Below listed connector types are required for Wall Outlet- TV and FM Radio connections.

- TV port: coaxial output MALE - EN500083-4 HD 134.2 S2 male IEC 169-2

- FM port: coaxial output FEMALE - EN500083-4 HD 134.2 S2 female IEC 169-2

## Environmental Requirements

### Packaging

- Wyre does not impose any requirements as this is the responsibility domain of the AO. However it is strongly advised to AO to follow industry standard practices

### RoHS and WEEE compliancy

- RoHS is defined as the directive on the restriction of the use of certain hazardous substances in electrical and electronic equipment on 2002/95/EC and the abbreviation for Restriction of Hazardous Substances. This directive is closely linked with Waste Electrical and Electronic Equipment Directive (WEEE) – 2002/96/EC.

- These directives are in the responsibility domain of AO, and Wyre does not impose any requirements. AO is strongly advised to follow the standards and the requirements imposed by law about RoHS.

### EU CoC compliancy

- Wyre does not impose any requirements as this is the responsibility domain of the AO.    However it is strongly advised to AO to follow industry standard practices and any requirements in this domain imposed by law should be met

## Safety Requirements

### Surge and Lightening protection

- Wyre requires that AO CPE devices are protected against over-voltages on their different interfaces. This is to guarantee customer safety under all conditions and to protect other CPE equipment.

### Temperature and Humidity

- Wyre does not impose any requirements as this is the responsibility domain of the AO. However it is strongly advised to AO to follow industry standard practices.

### Fire resistance

- Wyre requires AO to select equipment that has at least protection class 121, and has all parts halogen free / self-extinguishing.

- It should be noted however that being fully compliant with all legal requirements for CPE is the full and sole responsibility of the AO.

## EU Consumer Goods label Requirements

### CE - mark

- CE marking (originally EC mark) is a mandatory conformity mark for products placed on the market in the European Economic Area (EEA). With the CE marking on a product the manufacturer ensures that the product conforms to the essential requirements of the applicable EC directives. The letters "CE" stand for "Conformité Européenne" ("European Conformity").

(29) This conformity is in the responsibility domain of AO, and Wyre does not impose any requirements. AO is strongly advised to follow the standards and the requirements imposed by law.

# AO Equipment Wall Outlet Certification

## Introduction

- The Wall outlet is a termination point for AO CPE’s inbound coax cables. In some situations; such as NIU’s non-existence; WO will be a demarcation point for AO and Wyre interconnectivity on house-hold.

- Wall Outlet tests will cover all of the Functional Requirements, as part of TELENET-INTERKABEL certification.

## Test score card

Conformance Test Score Card Number

Test Identification

Test Execution Date

Test Run Type

Device / Equipment / Interface Name

Device / Equipment / Interface Type / Class

AO Device / Equipment / Interface Identification

Software Version

Tested by

Overall Result Status

Applicability

Test Cases Summary

FORMAT

IN SCOPE

MAN

PASS/FAIL

REM

"Y/N"

"P/F"

(*xy)

3.

Wall Outlet  Functional Requirements

HO

3.1.

RF Requirements

Y

3.2.

AO Device Management by Wyre Requirements

NA

4.

Wall Outlet Non Functional  Requirements

HO

4.1.

Mechanical Requirements

Y

4.2.

Environmental Requirements

Y

4.3.

Safety Requirements

Y

4.4.

EU Consumer Goods Label Requirements

Y

Remarks

CONFORMANCE TEST SCORE CARD

WRO-TA-TSC-%-P%%%

Full / Reduced

(without OOS cases)

CONFORMANCE TEST ITEM LIST

(*xy) : "Remark explanation comes here"

Pass / Fail

Select 1 or more : ROTV / ROBB / AIDTV