WRO Specification type Document

< Specification and Certification BB and VOD IP Interconnect >

Document Housekeeping

Document Category and type

| CAT | TYPE | DOC ID | Comment |
| --- | --- | --- | --- |
| Broadband | SPEC | WRO_TA_B_S_PAAB | Specification type documents (-SPEC) are documents specifying logical / physical interfaces / protocols, etc.., to which AO equipment/systems need to comply |

Legal Disclaimer

“This document constitutes an integral part of the Wyre Reference Offer for Basic TV / IDTV / BB and should be fully complied with by the Beneficiary at all times. Non compliance, incomplete or deviating application of this document by the Beneficiary, or his authorized agent, results in the suspension and ultimately termination of the Contract between Wyre and the Beneficiary.

At any time this document is susceptible to change by Wyre, Regulator’s decision or by decision of a relevant judicial authority. Changes to this document will, depending on the circumstances for change, be appropriately notified to the Beneficiary and published on the Wyre website.

Table of Contents

1	Abstract	5

2	AO IP Interconnect Solution Functional Description	6

3	AO IP Interconnect Solution Functional Requirements	7

3.1	General Architecture	7

3.2	Regional Interconnect Zones	8

3.2.1	Regional Interconnect Zones General Requirements	8

3.2.2	Regional Interconnect Zones Geographical View	8

3.2.3	Regional Interconnect Zones List of Head-Ends	9

3.2.4	Regional Interconnect Zones Traffic Routing Rules	11

3.3	AO Interconnect Links	12

3.3.1	AO Interconnect Links General Requirements	12

3.3.2	AO Interconnect Links Physical Connections	12

3.3.3	AO Interconnect Links Traffic Routing	12

3.3.4	AO Interconnect Links Redundancy	12

3.4	AO IP Range/Address Space	14

3.4.1	AO IP Range/Address Space General Requirements	14

3.4.2	AO IP Pool Management	14

3.5	Wholesale Broadband Network Gateway (WS-BNG)	15

3.5.1	WS-BNG General Requirements	15

3.5.2	WS-BNG Functions and Protocols	15

3.5.3	WS-BNG Accounting	15

3.6	AO Data and Management Links	15

3.6.1	AO Data Link	15

3.6.2	AO Management Link	15

3.6.3	AO VOD Link	15

3.7	AO Traffic Management	16

3.7.1	AO Traffic Management General Description	16

3.7.2	AO Traffic Management General Architecture	16

3.8	Restrictions	17

3.9	Operational Procedures	17

4	Certification for interconnection between AO IP backbone network and the Wyre network	18

4.1	Introduction	18

4.2	Test score card	18

Table of Figures

Figure 3-1: General Architecture	7

Figure 3-2: Regional Interconnect Zones	8

Figure 3-3: Physical Locations RPOI	9

Figure 3-4: List of Head-ends	9

Figure 3-5: Postal codes for Head-Ends	10

Figure 3-6: Postal codes for Head-Ends	10

Figure 3-7: Postal codes for Head-Ends	11

Figure 3-8: Traffic Management Architecture	15

List of Appendixes

This document may refer to further detailed documents that are added in Appendixes to this document.

A reference to an appendix is in this document highlighted with grey background.

Appendix A, <APP_B_S_PAAB_A> contains:

Appendix A – <AO Network Interface>

- The appendix (es) referred to in this section List of Appendixes, contain(s) detailed technical information which is only relevant when a Beneficiary enters in a concrete implementation project to become Beneficiary of the Wyre Reference Offer and/or Annex.

List of References

This document may refer to external documents or information sources.

A reference to an external document or information source is in this document highlighted with grey background.

The list of referred external documents or information sources in this document:

Reference 1: WRO_TA_G_C_PAAA - General Certification Procedures

Reference 2: WRO_TA_T_T_PAAA - Co-location and physical interconnect

Restricted information

This document may contain sections that are not public information and that can be made available only to parties that have executed specific NDA`s.

Information that is subject to NDA is marked in this document as follows:

The information in this text box is available only under NDA

NDA

NDA

Before conversion to PDF format for publication of the document, the information will be made unreadable by converting the background of the text box to black.

# Abstract

This document provides the requirement specifications to which the physical and logical level backbone network interconnections must comply in order to provide an IP interconnect system between the AO and Wyre IP networks carrying traffic in the data, control and OAM plane, supporting all services of the Wyre WRO. It also specifies routing, geographical traffic aspects, redundancy and fail-over considerations.

Generic sections specifying certification procedures applicable to all AO CPE or network equipment that will be connected to the Wyre network are described in General Certification Procedures Document WRO_TA_G_C_PAAA - General Certification Procedures.

# AO IP Interconnect Solution Functional Description

- This section describes the physical and logical backbone network interconnections on a functional level to which the solution must comply in order to provide an IP interconnect system between the AO and Wyre IP networks carrying traffic in the data, control and OAM plane, supporting all services of the Wyre WRO. It also specifies routing, geographical traffic aspects, redundancy and fail-over considerations.

- Further it also discusses traffic management policies on the interconnect links as well as handling regional aspects of AO end-user IP address allocation techniques.

# AO IP Interconnect Solution Functional Requirements

## General Architecture

Figure 3-1: General Architecture

- As shown in the figure above, AO CPE Traffic is transparently switched over the CMTS (L2VPN). From there the traffic is switched to a Wholesale Broadband Network Gateway (WS-BNG). The WS-BNG then routes traffic towards the correct AO interconnect link.

- There are two interconnects. A Data link for BB and a VOD link for VOD IP.

- From the WS-BNG all BB traffic is forced over that AO Data interconnect link. Traffic originating from the AO domain, and with as destination an end-user of the AO, will need to be routed towards the correct WS-BNG over the corresponding interconnect link. From the WS-BNG the traffic will be sent towards the AO end-user.

- The “AO-Data link” is used to carry AO CPE’s US/DS data and control plane traffic. The “AO-Mgmt-link”, which will be realized as an IP-VPN or IPSec type connection is used to carry management type of traffic like CDR file transfers, configuration file uploads, etc. between AO and Wyre.

- For VOD IP traffic between Wyre and the AO, the “AO-VOD link” is used. As described in the ROTV and AIDTV Architecture document.

## Regional Interconnect Zones

- There are 5 Regional Interconnect Zones (RIZ) which correspond to the location of the 5 Wyre switching Offices (SO). Those RIZ are identified by a list of HE which together constitutes a RIZ. Each HE serves a number of communities.

### Regional Interconnect Zones General Requirements

- Wyre operates in 5 Regional Interconnect Zones (RIZ) and can provide services to the end users in each zone. An AO must have interconnection to at least one of the five RPOI´s.

### Regional Interconnect Zones Geographical View

- An approximate Geographical coverage is view for the 5 Wyre RIZ can be seen in below figure.

Figure 3-2: Regional Interconnect Zones

- Also the physical Locations for 5 the RPOI can be seen in below figure.

- The addresses of the RPOI’s are respectively:

The information in this text box is available only under NDA

NDA

- NDA

Figure 3-3: Physical Locations RPOI

### Regional Interconnect Zones List of Head-Ends

- The lists of Head-Ends for each of the five Regional Interconnect Zones (ASSE, GENT, HASS, HOBO and ROES) are as shown below.

Figure 3-4: List of Head-ends

- Postal zip codes of communities belonging to the service area for each Head-End are shown below.

Figure 3-5: Postal codes for Head-Ends

Figure 3-6: Postal codes for Head-Ends

Figure 3-7: Postal codes for Head-Ends

### Regional Interconnect Zones Traffic Routing Rules

AO modems (or STB´s) will be unable to get service, if they are moved in another location than where they were initially installed. In case of a move for AO end-user Modem (or STB), the CM (or STB) needs to be re-registered in the Wyre NCP for the new specific region.

## AO Interconnect Links

### AO Interconnect Links General Requirements

- Wyre delivers all aggregated BB and/or VOD IP traffic of AO end-users towards AO via Wyre to AO interconnect link, distributed over 5 RPOI´s based on geographic origin of traffic.

- The interconnect for the aggregated BB and VOD IP traffic cannot be the same. The location for the interconnect for the aggregated BB and VOD IP traffic can, but must not be the same, selected by Wyre.

- Details over the interface are documented in Appendix A – <AO Network Interface>.

### AO Interconnect Links Physical Connections

- AO to Wyre interconnectivity is established in minimum 1 of the 5 regional POI´s (AO-RPOI) which are physically based in the Wyre switching office locations (Asse, Hoboken, Gent, Roeselare, Hasselt), where the WS-BNG will be located.  Wyre offers single or multiple 1Gbps, 10 Gbps or 100 Gbps link connections. If the link between AO and Wyre is a combination of multiple physical links, LAG support is required with LACP.

- The first RPOI the AO needs to connect to the RPOI in one of the regions to be selected by Wyre.

- Initially, there might be a single WS-BNG (possibly redundant) that will be used for all five regions. There will be a logical separation per region on the WS-BNG and the interconnect link.

There will be a restriction for the aggregated bandwidth of logical RPOI links. If the bandwidth for an interconnect exceed 10Gbps, Wyre reserves the right to request the AO to interconnect to additional RPOI’s.  The order in which the RPOI’s need to be added will be defined by Wyre taking into account the characteristics of the AO traffic resulting from the AO end-users. The purpose of adding RPOI’s should be to have the traffic spread out evenly throughout the Wyre network bearing in mind that offload of the AO traffic should be done as close to the source as possible.

- The different possibilities to realize the required physical link for the interconnect is described in the document WRO_TA_T_T_PAAA Co-location and physical interconnect.

### AO Interconnect Links Traffic Routing

- AO BB Traffic will be routed towards the AO backbone through Wyre to AO BB interconnect link. All traffic is forced over the AO interconnect link. A router redundancy protocol must be used for redundant interconnect links.

- Even if the destination of traffic is in the Wyre network, the BB traffic will always be routed to the AO network primarily and it will be the responsibility of the AO network to route it back to destination which is in Wyre network in this case.

- AO VOD IP traffic will be routed towards the AO backbone through Wyre to AO VOD interconnect link. Not all traffic is forced over the AO interconnect link. Refer to the ROTV and AIDTV Architecture document for the details of the traffic flows.

### AO Interconnect Links Redundancy

- Wyre offers link redundancy, based on redundant interface cards for AO interconnections to ensure that networks continue to function in the presence of single points of link failure.

## AO IP Range/Address Space

### AO IP Range/Address Space General Requirements

- Wyre does not offer IP address ranges for AO end-users. The AO has to provide sufficient IP address ranges for each of 5 RPOIs. Wyre needs to be informed about IP address ranges for each specific region. Wyre will request the required ranges the AO has to provide. The ranges need to contain sufficient IP addresses and be communicated upfront by the AO to Wyre and subject to appropriate change management as described in the relevant procedures in the Wyre WRO.

- For each update the AO has to provide minimum a /22 per required subnet for IPv4 ranges, and minimum a /64 per CPE subnet and a /40 for prefix delegation subnets for IPv6 ranges. There can only be a maximum of 4 updates per year. Each update can contain new subnets for multiple regions. The AO needs to request the implementation of new subnets at least 2 weeks in advance.

(*) Sufficient means the number of IP addresses per region that are needed to cover the AO end-users growth for minimal the next 6 months in order to limit the impact on the Wyre network and changes can be initiated in time.

- The AO will be required to have its own IP address range and IP address pools of sufficient size will have to be provided upfront to Wyre to allow proper routing set-up configuration in each of the connected RPOI´s, for the components detailed in the VOD IP section of the ROTV and AIDTV Architecture. The IP address ranges need to be uniquely aggregatable in one supernet for IPv4 and one for IPv6 over all the RPOI’s. For IPv4 if must be a public address range and for IPv6 a global unicast address range.

- On the AO customer’s premise, the STB must receive an IP address out of the same subnet as provided to that customer for the broadband offering, effectively making the STB part of the LAN. In order to both classify the VOD IP traffic and route it appropriately, to respect the PEP for VOD IP, the previous single IPv4 and IPv6 supernet requirement must be met. Also, the AO STB must use public IPv4 or a IPv6 global unicast address to connect to the components detailed in the VOD IP section of the ROTV and AIDTV Architecture. In case the AO uses Carrier Grade NAT, the STB should use IPv6.

### AO IP Pool Management

- Wyre participates in IP pool management and IP address assignment services for AO end-users. The effective IP address assignment to individual AO end-users customers is the responsibility of the AO NCP.  The AO uses its own IP address range for each of the 5 RPOIs and Wyre configures these ranges in the appropriate NE on a per RPOI basis. Multiple ranges are required per RPOI. The AO needs to assign IP addresses to its individual end-user devices that are selected from the AO IP ranges configured for a particular RPOI. The address “lease-time” needs to be of limited duration (maximum 24H). In addition, the “lease-time” should not be lower than 2 hours to avoid excessive network overhead unless requested otherwise during Planned Maintenances.

## Wholesale Broadband Network Gateway (WS-BNG)

### WS-BNG General Requirements

- The Gateway (WS-BNG) terminates AO end-user traffic which is sent over the Wyre access network using L2VPN BSoD technology.

- The L2VPN configuration for an AO modem is implemented via the modem configuration file.

### WS-BNG Functions and Protocols

- The Gateway (WS-BNG) functions as a transit router.  It receives IP data packets from the AO end-user devices and routes them to the AO network and vice versa.

### WS-BNG Accounting

- Wyre provides CDR type info about traffic usage of AO end-users for billing purpose. Implementing specific volume limits for individual end-users is the responsibility of AO.

## AO Data and Management Links

### AO Data Link

- The “AO Data Link” Interface between AO and Wyre is used to carry all the traffic originated from / destined to AO end-users.

### AO Management Link

- This interface is used for management purposes applications between AO and Wyre. Since the link carries sensitive traffic, it is secured by using IP-VPN or IPsec type connections.

### AO VOD Link

- This interface is used for connecting the Wyre provided services and components with the AO VOD Backend.

## AO Traffic Management

### AO Traffic Management General Description

- In order to guarantee a fair use of the available bandwidth in the network among all end-users Wyre performs traffic management to ensure that also in peak load situations end-users will have a good broadband service experience.

### AO Traffic Management General Architecture

Figure 3-8: Traffic Management Architecture

- Wyre will apply bandwidth management to both AO and Wyre end-users and potential other users equally.  Bandwidth limitations can be applicable on three levels:  HFC node level, aggregation level and optionally end-user level.

- HFC node level restrictions can be implemented on the PEP. PEP is the logical entity or place in the Wyre network that enforces policies for admission and bandwidth control. Practically it is implemented in different network elements. This type of traffic management is performed to avoid congestion and depends on the specific broadband product tier.

- Optionally, Wyre can apply bandwidth restriction on individual end-users based on their specific broadband product tier.

## Restrictions

- The broadband interconnection service explicitly excluded below listed services that belong to the AO domain as the ROBB service offered by Wyre is restricted to the delivery / pick-up of the traffic from/ to the interconnect links at the RPOI:

  - Value added services like (but not limited to): E-mail, Web space hosting, Virus scan, Personal firewalling, Wyre Hot-spot access, Wyre Home-spot access, Network based backup

  - My Wyre type subscription management functionality

  - Telemeter service

  - Re-direction towards landing page, offering volume block up-sell or choice for continue in narrowband services

  - AO end-user e-mail notification services on network events (e.g. high usage, …)

## Operational Procedures

- Wyre will execute from time to time operational changes on the IP interconnect infrastructure. An AO making use of the Wyre WRO part should be prepared at all time to adapt its infrastructure, devices and systems, as well as its operational procedures to handle those changes. In addition it is strongly recommended by Wyre that AO will take this into account in the design of its solution, so that impact of future changes will be limited. Below a non exhaustive list is given, showing some examples of operational changes that Wyre has executed in the past and which will be repeated likely in the future:

- Creation of new RIZ, or re-organisation of existing ones

- Updates and changes to the routing protocols and routing policies

- Updated and changes to the IP address allocation and sub-netting requirements

# Certification for interconnection between AO IP backbone network and the Wyre network

## Introduction

- The tests will cover all of the requirements specified by this document.

## Test score card

| CONFORMANCE TEST SCORE CARD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Conformance Test Score Card Number | WRO-TA-TSC-%-P%%% |  |  |  |  |
| Test Identification |  |  |  |  |  |
| Test Execution Date |  |  |  |  |  |
| Test Run Type | Full / Reduced(without OOS cases) |  |  |  |  |
| Device / Equipment / Interface Name |  |  |  |  |  |
| Device / Equipment / Interface Type / Class |  |  |  |  |  |
| AO Device / Equipment / Interface Identification |  |  |  |  |  |
| Software Version |  |  |  |  |  |
| Tested by |  |  |  |  |  |
| Overall Result Status | Pass / Fail |  |  |  |  |
| Applicability | Select 1 or more: ROTV / ROBB / AIDTV |  |  |  |  |
| CONFORMANCE TEST ITEM LIST |  |  |  |  |  |
| Test Cases Summary | FORMAT | IN SCOPE | MAN | PASS/FAIL | REM |
|  |  |  | "Y/N" | "P/F" | (*xy) |
| 3. AO IP Interconnect Solution Functional Requirements | HO |  |  |  |  |
| 3.1. General Architecture |  |  | Y |  |  |
| 3.2. Regional Interconnect Zones |  |  | Y |  |  |
| 3.3 AO Interconnect Links |  |  | Y |  |  |
| 3.4 AO IP Range / Address Space |  |  | Y |  |  |
| 3.5 Wholesale Broadband Network Gateway (WS-BNG) |  |  | Y |  |  |
| 3.6 AO Data and Management Links |  |  | Y |  |  |
| 3.7 AO Traffic Management |  |  | Y |  |  |
| 3.8 Restrictions |  |  | Y |  |  |
| 3.9 Operational Procedures |  |  | Y |  |  |
| Remarks |  |  |  |  |  |
| (*xy) : "Remark explanation comes here" |  |  |  |  |  |